# RealTech STP Automation - Server Interface Specification

Application version: 0.1.37
Protocol: MQTT 3.1.1 for JSON messages; SFTP for captured images
Default character encoding: UTF-8
Date/time format: ISO 8601 with timezone offset

## 1. Practical working summary

The gate application is local-first. RFID validation, balance checking, entry-fee deduction, image capture, trip creation, and barrier control continue even when the server is unavailable. Completed trips remain in SQLite with PendingSync status until they are uploaded.

### Device-to-server trip flow

- The IN or OUT lane reads an RFID card.

- The application finds the local vehicle/RFID master record.

- For a Paid IN trip, the current balance is checked and the configured entry fee is deducted.

- The balance change and trip record are saved in one SQLite transaction.

- The barrier sequence runs without waiting for the server.

- In the background, the image is uploaded by SFTP when available.

- The transaction JSON is published by MQTT with QoS 1.

- The trip is marked Synced after MQTT PUBACK is received.

### Server-to-device recharge flow

- The server publishes an RFID recharge command to the device-specific recharge topic.

- The application validates the JSON, site, device, RFID, vehicle, card type, and amount.

- The unique rechargeId is checked to prevent duplicate credit.

- The rechargeAmount is added to the latest local balance in a serialized SQLite transaction.

- A recharge audit row is stored in rfid_recharges.

- The application publishes an acknowledgement JSON.

- MQTT PUBACK is sent only after processing and acknowledgement publication complete. If a temporary failure occurs, reconnect/redelivery is safe because rechargeId is idempotent.

MQTTX is an MQTT client that can be used to test the interface. The production server can use MQTTX or any MQTT 3.1.1-compatible client/library.

## 2. Default MQTT topics

| Direction | Purpose | Default topic | QoS | Retain |
| --- | --- | --- | --- | --- |
| Device -> Server | Gate transaction | STP_COM | 1 | false |
| Server -> Device | RFID recharge command | STP_RECHARGE/STP-IWS-001 | 1 | false |
| Device -> Server | Recharge acknowledgement | STP_RECHARGE_ACK/STP-IWS-001 | 1 | false |

Use a separate recharge topic and subscriber client ID for every installed device. Do not connect MQTTX using the same client ID as the application because an MQTT broker normally permits only one active connection per client ID.

## 3. Gate transaction JSON: device to server

### Sample normal IN transaction

{
  "schemaVersion": "1.1",
  "transactionId": "8d1d31e9-9a38-472c-a3a0-df88f5bc52e7",
  "siteId": "SITE-001",
  "deviceId": "Tambaram STP 001",
  "laneId": "MAIN-GATE",
  "deviceName": "STP-IWS-001",
  "direction": "IN",
  "rfidNumber": "E20034120123456789012345",
  "vehicleNumber": "TN11AB1234",
  "accessType": "Paid",
  "entryTransactionId": null,
  "previousBalance": 500.00,
  "debitAmount": 50.00,
  "newBalance": 450.00,
  "imageCaptureStatus": "Captured",
  "imageFileName": "Tambaram STP Site 1 - 2026-07-22_10-30-15-125 - IN - 8d1d31e9-9a38-472c-a3a0-df88f5bc52e7.png",
  "imageRemotePath": "/home/ftpuser/ftp/STP/Tambaram/STP-IWS-001/Tambaram STP Site 1 - 2026-07-22_10-30-15-125 - IN - 8d1d31e9-9a38-472c-a3a0-df88f5bc52e7.png",
  "tripDate": "2026-07-22",
  "tripTime": "10:30:15.125",
  "tripTimestamp": "2026-07-22T10:30:15.125+05:30",
  "tripTimestampUtc": "2026-07-22T05:00:15.125+00:00",
  "timeZoneOffset": "+05:30",
  "processedAt": "2026-07-22T10:30:15.125+05:30",
  "notes": "Processed locally; server upload is asynchronous."
}

### Transaction field details

| Field | Type | Required | Meaning |
| --- | --- | --- | --- |
| schemaVersion | string | yes | Current transaction schema is 1.1. |
| transactionId | UUID string | yes | Unique local trip ID; server should process idempotently. |
| siteId | string | yes | Site identifier from device configuration. |
| deviceId | string | yes | Device/controller identifier. |
| laneId | string | yes | Logical gate/lane identifier. |
| deviceName | string | yes | Human-readable device name. |
| direction | string | yes | IN or OUT. |
| rfidNumber | string | yes | Normalized uppercase RFID/EPC without whitespace. |
| vehicleNumber | string | yes | Registered vehicle number. |
| accessType | string | yes | Free or Paid. |
| entryTransactionId | UUID/null | no | OUT trip's matching IN trip ID. |
| previousBalance | decimal | yes | Balance immediately before this local transaction. |
| debitAmount | decimal | yes | Entry fee deducted; normally zero for OUT and Free access. |
| newBalance | decimal | yes | Balance after the local transaction. |
| imageCaptureStatus | string | yes | Capture result such as Captured or an error status. |
| imageFileName | string/null | no | Local/remote image filename. |
| imageRemotePath | string/null | no | SFTP path after successful upload. |
| tripDate | string | yes | Local date, yyyy-MM-dd. |
| tripTime | string | yes | Local time with milliseconds. |
| tripTimestamp | ISO timestamp | yes | Local timestamp with offset. |
| tripTimestampUtc | ISO timestamp | yes | UTC timestamp. |
| timeZoneOffset | string | yes | Offset such as +05:30. |
| processedAt | ISO timestamp | yes | Backward-compatible local processed timestamp. |
| notes | string | yes | Processing/reconciliation notes. |

### Exceptional approval extension

For missing-IN or missing-OUT reconciliation, the payload also contains:

{
  "exceptionalApproval": {
    "reason": "Missing IN Trip",
    "approverName": "Supervisor Name",
    "approverRole": "Shift Supervisor",
    "approverMobile": "9876543210",
    "approvalDate": "2026-07-22",
    "approvalTime": "10:31:12.450",
    "approvedAt": "2026-07-22T10:31:12.450+05:30",
    "approvedAtUtc": "2026-07-22T05:01:12.450+00:00",
    "timeZoneOffset": "+05:30"
  }
}

## 4. RFID recharge command: server to device

### Sample recharge command

Publish this JSON to STP_RECHARGE/STP-IWS-001 with QoS 1 and retain disabled:

{
  "schemaVersion": "1.0",
  "messageType": "rfidRecharge",
  "rechargeId": "RCH-20260722-000001",
  "siteId": "SITE-001",
  "deviceId": "Tambaram STP 001",
  "rfidNumber": "E20034120123456789012345",
  "vehicleNumber": "TN11AB1234",
  "previousBalance": -50.00,
  "rechargeAmount": 500.00,
  "newBalance": 450.00,
  "rechargedAt": "2026-07-22T10:35:00+05:30",
  "paymentReference": "PAY-20260722-77881",
  "operatorId": "OP-1007",
  "operatorName": "Recharge Operator",
  "remarks": "Cash recharge at site office"
}

### Recharge field details

| Field | Type | Required | Validation/usage |
| --- | --- | --- | --- |
| schemaVersion | string | recommended | Use 1.0. |
| messageType | string | yes | Must be rfidRecharge. |
| rechargeId | string | yes | Globally unique, maximum 100 characters; idempotency key. |
| siteId | string | recommended | When present, must match the device siteId. |
| deviceId | string | recommended | When present, must match the device deviceId. |
| rfidNumber | string | yes | RFID/EPC; whitespace is removed and value is uppercased. |
| vehicleNumber | string | optional | When present, must match the vehicle registered to the RFID. |
| previousBalance | decimal | optional | Server's balance before recharge; used for payload consistency/reconciliation. |
| rechargeAmount | decimal | yes | Positive amount to add to the latest local balance. |
| newBalance | decimal | optional | Server's expected balance after recharge. |
| rechargedAt | ISO timestamp | optional | Time the recharge occurred on the server. |
| paymentReference | string | optional | Payment/receipt/reference ID. |
| operatorId | string | optional | Server/recharge operator ID. |
| operatorName | string | optional | Server/recharge operator name. |
| remarks | string | optional | Free-text recharge note. |

When all three balance fields are present, the message is rejected if previousBalance + rechargeAmount differs from newBalance by more than 0.01.

### Balance conflict policy

The application applies rechargeAmount as a delta to the latest local balance. It does not blindly overwrite the balance with newBalance. This is intentional because gate transactions are processed locally while the server can be offline. A delayed server snapshot could otherwise erase a local debit.

The acknowledgement contains the actual local before/after balances and a serverBalanceMatched flag. The server should reconcile when this flag is false.

## 5. Recharge acknowledgement: device to server

### Successful application

{
  "schemaVersion": "1.0",
  "messageType": "rfidRechargeAck",
  "rechargeId": "RCH-20260722-000001",
  "siteId": "SITE-001",
  "deviceId": "Tambaram STP 001",
  "rfidNumber": "E20034120123456789012345",
  "vehicleNumber": "TN11AB1234",
  "status": "Applied",
  "success": true,
  "duplicate": false,
  "rechargeAmount": 500.00,
  "previousBalance": -50.00,
  "newBalance": 450.00,
  "serverExpectedNewBalance": 450.00,
  "serverBalanceMatched": true,
  "processedAt": "2026-07-22T10:35:01.240+05:30",
  "message": "Recharge applied to the local RFID balance."
}

### Duplicate command

A repeated rechargeId is never credited twice:

{
  "schemaVersion": "1.0",
  "messageType": "rfidRechargeAck",
  "rechargeId": "RCH-20260722-000001",
  "siteId": "SITE-001",
  "deviceId": "Tambaram STP 001",
  "rfidNumber": "E20034120123456789012345",
  "vehicleNumber": "TN11AB1234",
  "status": "Duplicate",
  "success": true,
  "duplicate": true,
  "rechargeAmount": 500.00,
  "previousBalance": -50.00,
  "newBalance": 450.00,
  "serverExpectedNewBalance": 450.00,
  "serverBalanceMatched": true,
  "processedAt": "2026-07-22T10:35:01.240+05:30",
  "message": "Recharge ID was already processed with status Applied: Recharge applied to the local RFID balance."
}

### Rejected command

{
  "schemaVersion": "1.0",
  "messageType": "rfidRechargeAck",
  "rechargeId": "RCH-20260722-000002",
  "siteId": "SITE-001",
  "deviceId": "Tambaram STP 001",
  "rfidNumber": "E20000000000000000000000",
  "vehicleNumber": "",
  "status": "Rejected",
  "success": false,
  "duplicate": false,
  "rechargeAmount": 500.00,
  "serverExpectedNewBalance": 500.00,
  "processedAt": "2026-07-22T10:36:10.100+05:30",
  "message": "RFID is not registered."
}

Possible rejection reasons include invalid message type, missing RFID, non-positive amount, wrong site/device, inconsistent balance arithmetic, unregistered RFID, inactive RFID, Free RFID, or vehicle number mismatch.

## 6. MQTT configuration sample

{
  "Server": {
    "Enabled": true,
    "SyncIntervalSeconds": 15,
    "MaxBatchSize": 25,
    "Mqtt": {
      "BrokerHost": "mqtt.example.com",
      "Port": 1883,
      "UseTls": false,
      "Username": "<username>",
      "Password": "<password>",
      "ClientId": "STP-IWS-001",
      "BaseTopic": "STP_COM",
      "PublishTopic": "STP_COM",
      "QualityOfService": 1,
      "Retain": false,
      "KeepAliveSeconds": 30,
      "ConnectTimeoutSeconds": 10,
      "PublishTimeoutSeconds": 10,
      "RechargeSyncEnabled": true,
      "RechargeSubscribeTopic": "STP_RECHARGE/STP-IWS-001",
      "RechargeAckTopic": "STP_RECHARGE_ACK/STP-IWS-001",
      "RechargeSubscriberClientId": "STP-IWS-001-recharge",
      "RechargeReconnectSeconds": 5
    }
  }
}

Changing Server Panel settings requires restarting the application so hosted MQTT workers reconnect with the new configuration.

## 7. MQTTX test procedure

- Start the RealTech STP Automation application and confirm the server log reports that the recharge subscriber is connected.

- In MQTTX, create a broker connection using a unique client ID such as MQTTX-TEST-001.

- Subscribe in MQTTX to STP_RECHARGE_ACK/STP-IWS-001 using QoS 1.

- Publish the recharge command sample to STP_RECHARGE/STP-IWS-001 using QoS 1 and retain false.

- Verify an Applied acknowledgement is received.

- Publish the exact same JSON again. Verify status is Duplicate and the card is not credited again.

- Scan the RFID at the IN lane and verify the displayed/debited balance uses the recharged value.

- Publish commands with an unknown RFID, Free RFID, wrong device ID, zero amount, and mismatched vehicle number. Verify Rejected acknowledgements.

- Disconnect the application after it has subscribed, publish a QoS 1 message, then reconnect. Verify queued delivery if the broker supports persistent sessions.

## 8. Server retry rule

The server should keep a recharge in Pending state until it receives a matching acknowledgement by rechargeId. On timeout it may safely republish the same command with the same rechargeId. It must not generate a new recharge ID for a retry, because a new ID represents a new credit operation.

## 9. Security and production notes

- Use TLS (UseTls: true, normally port 8883) when supported by the broker.

- Use broker ACLs so each device can subscribe only to its recharge topic and publish only to its transaction and acknowledgement topics.

- Do not use production application client IDs in MQTTX.

- Do not enable retained recharge messages. Use QoS 1 plus acknowledgement/retry.

- Store broker and SFTP credentials with restricted file permissions.

- The application records applied recharges and RFID/master-data rejection results, including the original JSON, in SQLite for audit.

## 10. Current acknowledgement limitation for gate transactions

Gate transaction synchronization currently treats MQTT QoS 1 PUBACK as delivery success. It does not wait for a separate server/application acknowledgement for STP_COM. The recharge interface does have an application-level acknowledgement because balance credit must be explicitly confirmed.