cat > /tmp/fix_usb_audio.sh <<'BASH'

#!/usr/bin/env bash

set -euo pipefail

sudo systemctl stop gcam.service || true

echo "Playback devices:"

aplay -l

echo "Setting USB audio as default: hw:0,0"

sudo tee /etc/asound.conf > /dev/null <<'EOF'

pcm.!default {

type plug

slave.pcm "hw:0,0"

}

ctl.!default {

type hw

card 0

}

EOF

amixer -c 0 set Speaker 100% unmute || true

amixer -c 0 set PCM 100% unmute || true

amixer -c 0 set Master 100% unmute || true

sudo pkill -9 aplay || true

sudo pkill -9 ffplay || true

sleep 2

echo

echo "Testing USB speaker with speaker-test..."

speaker-test -D plughw:0,0 -c 2 -t wav -l 1 || true

echo

echo "Testing USB speaker with warning.wav..."

aplay -D plughw:0,0 /opt/gcam/assets/warning.wav

echo

echo "Testing default ALSA device..."

aplay /opt/gcam/assets/warning.wav

sudo systemctl restart gcam.service

echo

echo "Done. Current /etc/asound.conf:"

cat /etc/asound.conf

BASH

chmod +x /tmp/fix_usb_audio.sh

bash /tmp/fix_usb_audio.sh

cp /tmp/warning.wav /opt/gcam/assets/warning.wav

aplay /opt/gcam/assets/warning.wav

sudo systemctl restart gcam.service

sudo nano /boot/firmware/config.txt

arm_freq=2000

usb_max_current_enable=1

Cntl + O

Enter

Cntl + X

sudo rpi-eeprom-config --edit

PSU_MAX_CURRENT=5000

Cntl + O

Enter

Cntl + X

sudo reboot