UHF Senior Module User Guide

Contents

1 Docking communication protocol	2

1.1 Serial communication parameters	2

1.2 Data frame format	2

See Appendix B for CRC16 reference codes.	4

2 Command frame set	4

2.1 Command list	4

2.2 Module common control commands	6

2.2.0 RFM__MODULE_INIT	6

2.2.1 RFM_GET_INFO	6

2.2.2 RFM_REBOOT	7

2.2.3 RFM_SET_PWR	7

2.2.4 RFM_GET_PWR	7

2.2.5 RFM_SET_FRE	8

2.2.6 RFM_GET_FRE	10

2.2.7 RFM_SET_ANTENNA	10

2.2.8 RFM_GET_ANTENNA	10

2.2.9 RFM_SET_GET_RFID	11

2.2.10 RFM_SET_GET_BAUD	12

2.2.11 RFM_SET_GET_ BUZZER	13

2.2.12 RFM_SET_GET_ ADDR	13

2.2.13 RFM_GET_SET_PARA	14

2.2.16 RFM_MAXTEMPER_SET	16

2.2.17 RFM_CURRTEMPER_GET	16

2.2.19 RFM_SET_GET_ANT_POWER	17

2.2.21 RFM_SET_GET_DUTY_CYCLE	18

2.2.22 RFM_SET_ALL_PARAM	18

2.2.23 RFM_GET_ALL_PARAM	24

2.3 International standard (ISO 18000-6C) protocol-related commands	25

2.3.1 RFM_INVENTORYISO_CONTINUE	25

2.3.2 RFM_INVENTORY_STOP	26

2.3.3 RFM_READISO_TAG	27

2.3.4 RFM_WRITEISO_TAG	28

2.3.5 RFM_LOCKISO_TAG	29

2.3.6 RFM_KLENLISO_TAG	30

2.3.7 RFM_SETISO_SELECTMASK	31

2.3.8 RFM_SET_ISO_Q	32

2.3.9 RFM_GET_ISO_Q	32

2.3.10 RFM_SET_SELPRM	33

2.3.11 RFM_GET_SELPRM	34

2.3.12 RFM_SET_QUERY_PARAM	36

2.3.13 RFM_GET_QUERY_PARAM	37

2.3.15 RFM_INVENTORYISO_RANGE_DATA	38

Appendix A. Operational Status Returned by Tags	39

Appendix B. CRC16 check reference C code	40

Appendix C. Definition of STATUS。	40

1 Docking communication protocol

This module can communicate with external applications through the serial port 232 TTL.

1.1 Serial communication parameters

The module communicates with external applications through the serial port

Table A-1 Module docking serial port parameters

| Project | Parameter | Remark |
| --- | --- | --- |
| baud rate | 115200bps |  |
| data bit | 8 |  |
| stop bit | 1 |  |
| parity bit | None |  |

1.2 Data frame format

The data packet issued by the upper application is hereinafter referred to as "command", and the data packet returned by the module to the upper application is hereinafter referred to as "response". The length unit of all data segments below is byte.

Data transmission sequence: For each data item consisting of multiple bytes, the most significant byte is sent first, and the least significant bit is sent first within the byte.

The data frame format of the command is shown in Table A-2:

Table A-2 Data frame format of the command

| MSB |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- |
|  | Control Field |  |  | Information Field | EpLENogue Field |
| HEAD | ADDR | CMD | LEN | Data[] | CHECK |
| 1Byte | 1Byte | 2Bytes | 1Byte | 0~255Byte | 2Byte |

The format of the response data frame is shown in Table A-3:

Table A-3 Response data frame format

| MSB |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Control Field |  | Information Field |  | EpLENogue Field |
| HEAD | ADDR | CMD | LEN | STATUS | Data[] | CHECK |
| 1Byte | 1Byte | 2Bytes | 1Byte | 1Byte | 0~254Byte | 2Byte |

The data frame structure includes the following three parts: Control Field、Information Field and EpLENogue Field.

|  |  | length (byte) | Explanation |
| --- | --- | --- | --- |
| Control Field | HEAD | 1 | HEAD is fixed as the hexadecimal number 0xCF, and this byte is used for data frame synchronization. |
|  | ADDR | 1 | Reader address. Address range: 0x00~0xFE, 0xFF is the broadcast address, and the reader only responds to commands that are the same as its own address and whose address is 0xFF. The address of the reader is 0x00 when it leaves the factory. |
|  | CMD | 2 | For the command code and command word definition, see Table A-7. |
|  | LEN | 1 | This length is the data length of all bytes of the actual Information Field, and the maximum data length is 255Bytes (0xFF). |
| Information Field | STATUS | 1 | STATUS is the execution status of the host computer command contained in the module's response. STATUS only exists in the response data frame returned by the module, and there is no STATUS part in the command frame sent by the upper layer application. If the STATUS is 0, it means that the command is successfully executed. The successful execution here only means that the module has successfully received the tag response. If there is a tag execution status in the tag response, it should be further judged whether the tag execution status is correct. See Table A-6 for the definition of STATUS. |
|  | Data[] | indefinite | parameter field. In the actual command, it may not exist. Data[] is the actual data that needs to be transmitted, and the effective bytes of Data[] will be specifically defined in each command format. |
| EpLENogue Field | MSB-CRC16 | 1 | CRC16 high byte. CRC16 is the CRC16 value from HEAD to Data[] |
|  | LSB-CRC16 | 1 | CRC16 low byte. |

See Appendix B for CRC16 reference codes.

2 Command frame set

The UHF RFID reader module is referred to as "module" for short in the following description.

2.1 Command list

表格A-7控制命令示例列表

| Command name | CMD control byte | Functional description | Remark |
| --- | --- | --- | --- |
| EPC instruction |  |  |  |
| RFM_INVENTORYISO_CONTINUE | 0x0001 | Tag inventory (process) |  |
| RFM_INVENTORYISO_STOP | 0x0002 | stop tag counting |  |
| RFM_READISO_TAG | 0x0003 | Read tag data (process) |  |
| RFM_WRITEISO_TAG | 0x0004 | Write tag data (process) |  |
| RFM_LOCKISO_TAG | 0x0005 | Lock Tag Data (Process) |  |
| RFM_KLENLISO_TAG | 0x0006 | Kill tags (process) |  |
| RFM_SETISO_SELECTMASK | 0x0007 | The tag specified when setting the operation label |  |
| RFM_SET_ISO_Q | 0x0008 | Set protocol Q value |  |
| RFM_GET_ISO_Q | 0x0009 | Get protocol Q value |  |
| RFM_SET_SELPRM | 0x0010 | Select parameter setting |  |
| RFM_GET_SELPRM | 0x0011 | Select parameter acquisition |  |
| RFM_SET_QUERY_PARAM | 0x0012 | Query parameter settings |  |
| RFM_GET_QUERY_PARAM | 0x0013 | Query parameter acquisition |  |
| RFM_INVENTORYISO_RANGE_DATA | 0x0018 | Specifies the output data of the inventory |  |
| Module Custom Command |  |  |  |
| RFM_MODULE_INIT | 0x0050 | Initialize device (stop all actions) |  |
| RFM_GET_INFO | 0x0051 | Obtain module version information and sn code |  |
| RFM_REBOOT | 0x0052 | reset |  |
| RFM_SET_PWR | 0x0053 | Set the RF output power of the module |  |
| RFM_GET_PWR | 0x0054 | Read module RF output power |  |
| RFM_SET_FRE | 0x0055 | Set the module RF operating frequency |  |
| RFM_GET_FRE | 0x0056 | Read module RF operating frequency |  |
| RFM_SET_ANTENNA | 0x0057 | set up the antenna |  |
| RFM_GET_ANTENNA | 0x0058 | read antenna status |  |
| RFM_SET_GET_RFID | 0x0059 | Set and read RFID protocol standard |  |
| RFM_SET_GET_BAUD | 0x005A | Set/read current baud rate |  |
| RFM_SET_GET_BUZZER | 0x005B | set/read buzzer |  |
| RFM_SET_GET_ADDR | 0x005C | Set/read device address |  |
| RFM_SET_PARA | 0x005D | Set device parameters |  |
| RFM_SET_GET_NETPARA | 0x005F | Set to get and set device network port information |  |
| RFM_MAXTEMPER_SET | 0x0060 | Set the maximum operating temperature |  |
| RFM_CURRTEMPER_GET | 0x0061 | Get current working temperature |  |
| RFM_SET_GET_ANT_POWER | 0x0063 | Set different antenna power |  |
| RFM_SET_GET_DUTY_CYCLE | 0x0065 | Set Get/Set Inventory Duty Cycle |  |

2.2 Module common control commands

2.2.0 RFM__MODULE_INIT

This command is used to initialize the module and stop all current actions.

- Command format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x0050 | 0x00 | 2Byte |

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0050 | 0x01 | 1Byte | 2Byte |

Byte definition:

| STATUS | 1 | Return command status value |
| --- | --- | --- |

2.2.1 RFM_GET_INFO

This command is used to obtain the software and hardware version information of the current module.

- Command format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x0051 | 0x00 | 2Byte |

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS |  | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | HardVer | FirmVer | SN_code |  |
| 0xCF | 0x00 | 0x0051 | 0x5C | 1Byte | 32Bytes | 32Bytes | 12Bytes | 2Byte |

Definition of each byte：

| field | byte | describe |
| --- | --- | --- |
| STATUS | 1 | This command will only return successful execution (value 0x00), other values are invalid; |
| HardVer | 32 | The software and hardware model and version number are ASCII codes, ending with '\0'. The actual software and hardware version number may be less than 32 bytes, and other bytes that are not used are supplemented with '\0'; |
| FirmVer | 32 | 32-byte software model and bootloader version number, the format is the same as the hardware version number; |
| SN_code | 12 | 12-byte serial number, the format is ASCII code data, other bytes not used are supplemented with '\0'; |

2.2.2 RFM_REBOOT

Restore factory settings command, when the command is executed successfully, the module will restore factory settings (does it need to be restarted?).

- Command format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x0052 | 0x00 | 2Byte |

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0052 | 0x01 | 1Byte | 2Byte |

 STATUS：This command will only return successful execution (value 0x00), other values are invalid;

2.2.3 RFM_SET_PWR

This command is used to set the RF output power of the module. Before using the module to operate the tag, the user needs to use this command to set the RF output power of the module. If the user does not set the RF output power of the module, the module will use the default setting when working.

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Power | Resv |  |
| 0xCF | 0xFF | 0x0053 | 0x02 | 1Byte | 1Byte | 2Byte |

  - Power：Output power, unit: dBm, value range: [10, 33]dBm, others are invalid

  - Resv：Reserved value: default 0x00;

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0053 | 0x01 | 1Byte | 2Byte |

  - STATUS：

  - 0x00：The command was executed successfully;

  - 0x01：The module does not support the output power value;

  - Other values: void;

2.2.4 RFM_GET_PWR

- This command is used to read the RF power currently set by the module. Users can use this command to view the power setting of the module before using the module to operate the label.

- command format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x0054 | 0x00 | 2Byte |

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Power | Resv |  |
| 0xCF | 0x00 | 0x0054 | 0x03 | 1Byte | 1Byte | 1Byte | 2Byte |

  - STATUS：This command will only return successful execution (value 0x00), other values are invalid;

  - Power：Output power, unit: dBm, value range: [10, 33]dBm, others are invalid;

  - Resv：System reserved fields;

2.2.5 RFM_SET_FRE

This command is used to set the RF output frequency of the module. There are two working modes of the module: frequency hopping mode and fixed frequency mode. The frequency hopping working mode of the module is determined by the number of channels of the set working frequency. If the number of channels is greater than 1, it means that the module is set to work in frequency hopping mode. The number of channels is equal to 1 indicates that the module is set to work in fixed frequency mode.

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | REGION | STRATFREI | STRATFRED | STEPFRE | CN |  |
| 0xCF | 0xFF | 0x0055 | 1Byte | 1 Byte | 2Byte | 2Byte | 2Byte | 1Byte | 1Byte |

Definition of each byte:

To set the frequency, first determine the frequency range according to REGION, and then refer to STRATFREI, STRATFRED, STEPFRE, CN to calculate the specific frequency range.

| 字段 | byte | describe |
| --- | --- | --- |
| REGION | 1 | National frequency range: 0x00: User-defined according to requirements; 0x01：US [902.75~927.25] 0x02：Korea [917.1~923.5] 0x03：EU [865.1~868.1] 0x04：JAPAN [952.2~953.6] 0x05：MALAYSIA [919.5~922.5] 0x06：EU3 [865.7~867,5] 0x07：CHINA_BAND1 [840.125~844.875] 0x08：CHINA_BAND2 [920.125~924.875] |
| STRATFREI | 2 | The integer part of the megahertz start frequency; such as 920.125MHz, STRATFREI = 920 = 0x398, high byte = 0x03, low byte = 0x98; |
| STRATFRED | 2 | The fractional part of the megahertz starting frequency; such as 920.125MHz, STRATFRED=125, high byte=0x00, low byte=0x7D |
| STEPFRE | 2 | Frequency step (KHz), need to refer to the calculation formula of each frequency band; such as 125KHz, STEPFRE =125, high byte=0x00, low byte=0x7D; |
| CN | 1 | number of channels; |

Frequency calculation formula:

Minimum frequency point: Fmin = STRATFREI + STRATFRED/1000 (unit: MHz)

Maximum frequency point: Fmax = Fmin + STEPFRE*CN/1000 (unit: MHz)

Calculation formula for each frequency band:(The set frequency range must be within the national standard)

Chinese band2：		Fs = 920.125 + CN * 0.25 (MHz) where CN∈[0, 19].

US band：				Fs = 902.75 + CN * 0.5 (MHz) where CN∈[0,49]。

Korean band：			Fs = 917.1 + CN * 0.2 (MHz) where CN∈[0, 31]。

EU band:				Fs = 865.1 + CN*0.2(MHz) where CN∈[0, 14]。

Ukraine band:			Fs = 868.0 + CN*0.1(MHz) where CN∈[0, 6]。

Peru band：			Fs = 916.2 + CN*0.9(MHz) where CN∈[0, 11]。

Chinese band1：		Fs = 840.125 + CN * 0.25 (MHz) where CN∈[0, 19]。

EU3 band：			Fs = 865.7 + CN * 0.6(MHz) where CN∈[0, 3]。

US band3：			Fs = 902 + CN * 0.5 (MHz) where CN∈[0,52]。

Taiwan band：			Fs = 922.25 + CN * 0.5 (MHz) where CN∈[0,11]。

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0055 | 0x01 | 1Byte | 2Byte |

  - STATUS：

  - 0x00：command executed successfully；

  - 0x01：Parameter error；

  - Other values: invalid；

2.2.6 RFM_GET_FRE

This command is used to read the RF working output frequency of the module.

- command format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x0056 | 0x00 | 2Byte |

- 响应格式及状态字节

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | REGION | STRATFREI | STRATFRED | STEPFRE | CN |  |
| 0xCF | 0x00 | 0x0056 | 1Byte | 1Byte | 1 Byte | 2Byte | 2Byte | 2Byte | 1Byte | 2Byte |

The definition of each byte refers to RFM_SET_FRE (set RF working frequency)

2.2.7 RFM_SET_ANTENNA

This command is used to manually set the on-site antenna used by the module. This command is generally used for the module to automatically find the situation where the on-site antenna fails, and manually intervene to set the on-site antenna.

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD | CHECK |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Antenna |  |
| 0xCF | 0xFF | 0x0057 | 0x01 | 1Byte | 2Byte |

  - Antenna：Indicates the selected antenna by bit. The value of the corresponding Bit bit is 1, which means that the antenna is used, and the value is 0, which means that the antenna is not used; starting from the lower bit, the 0th bit indicates the antenna 1, and the 1st bit indicates the antenna 2. By analogy, up to 8 antennas can be represented, and different modules support different antennas, depending on the specific situation;

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Antenna |  |
| 0xCF | 0x00 | 0x0057 | 0x02 | 1Byte | 1Byte | 2Byte |

  - STATUS：

  - 0x00：The command was executed successfully;

  - 0x01：Parameter error;

  - Other values: void;

2.2.8 RFM_GET_ANTENNA

This command is used to read the antenna used by the module.

- command format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x0058 | 0x00 | 2Byte |

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Antenna |  |
| 0xCF | 0x00 | 0x0058 | 0x02 | 1Byte | 1Byte | 2Byte |

  - STATUS：This command will only return successful execution (value 0x00), other values are invalid;

  - Antenna：Indicates the selected antenna by bit, see the description of RFM_SET_ANTENNA for Antenna;

2.2.9 RFM_SET_GET_RFID

- This command sets/reads the protocol standard specification of the radio frequency RFID of the module (the protocol standard can be set by issuing this command to determine whether the module is connected successfully).

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | RFIDPRO |  |
| 0xCF | 0xFF | 0x0059 | 1Byte | 1Byte | NC or 1Byte | 2Byte |

  - Option：command control options

0x01：set up;

0x02：read;

Other values: void;

- Response format and status byte

Set response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0059 | 1Byte | 1Byte | 1Byte | 2Byte |

get response：

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | RFIDPRO |  |
| 0xCF | 0x00 | 0x0059 | 1Byte | 1Byte | 1Byte | NC or 1Byte | 2Byte |

| field | byte | describe |
| --- | --- | --- |
| STATUS | 1 | 0x00: The command is executed successfully; 0x01: The parameter is wrong. |
| Option | 1 | 0x01: set; 0x02: get |
| RFIDPRO | 1 | 0x00: ISO 18000-6C; 0x01: GB/T 29768; 0x02: GJB 7377.1; currently only supports ISO 18000-6C. |

2.2.10 RFM_SET_GET_BAUD

This command sets/reads the baud rate of the module

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | BAUD |  |
| 0xCF | 0xFF | 0x005A | 1Byte | 1Byte | NC or 1Byte | 2Byte |

  - Option：command control options

0x01：set up；

0x02：read；

Other values: invalid；

- Response format and status byte

set response：

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x005A | 1Byte | 1Byte | 1Byte | 2Byte |

get response：

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | BAUD |  |
| 0xCF | 0x00 | 0x0005A | 1Byte | 1Byte | 1Byte | NC or 1Byte | 2Byte |

| field | byte | describe |
| --- | --- | --- |
| STATUS | 1 | 0x00: The command is executed successfully; 0x01: The parameter is wrong. |
| Option | 1 | 0x01: set; 0x02: get |
| BAUD | 1 | 0x00：9600； 0x01：19200；0x02：38400；0x03：57600；0x04：115200(default)； |

2.2.11 RFM_SET_GET_ BUZZER

This command sets/reads the buzzer status of the module

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | BUZZER |  |
| 0xCF | 0xFF | 0x005B | 1Byte | 1Byte | NC or 1Byte | 2Byte |

  - Option：command control options

0x01：set up；

0x02：read；

Other values: invalid；

- Response format and status byte

set response：

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x005B | 1Byte | 1Byte | 1Byte | 2Byte |

get response：

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | BUZZER |  |
| 0xCF | 0x00 | 0x0005B | 1Byte | 1Byte | 1Byte | NC or 1Byte | 2Byte |

| field | byte | describe |
| --- | --- | --- |
| STATUS | 1 | 0x00: The command is executed successfully; 0x01: The parameter is wrong. |
| Option | 1 | 0x01: set; 0x02: get |
| BUZZER | 1 | 0x00: Close (default); 0x01: Open; |

2.2.12 RFM_SET_GET_ ADDR

This command sets/reads the address of the module

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | ADDR |  |
| 0xCF | 0xFF | 0x005C | 1Byte | 1Byte | NC or 1Byte | 2Byte |

  - Option：command control options

0x01：set up；

0x02：read；

Other values: invalid；

- Response format and status byte

set response：

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x005C | 1Byte | 1Byte | 1Byte | 2Byte |

get response：

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | ADDR |  |
| 0xCF | 0x00 | 0x0005C | 1Byte | 1Byte | 1Byte | NC or 1Byte | 2Byte |

| field | byte | describe |
| --- | --- | --- |
| STATUS | 1 | 0x00: The command is executed successfully; 0x01: The parameter is wrong. |
| Option | 1 | 0x01: set; 0x02: get |
| ADDR | 1 | 0x00~0xFF, the default is 0x00 |

2.2.13 RFM_GET_SET_PARA

This command is used to get setting module parameters.

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | ADDR |  |
| 0xCF | 0xFF | 0x005D | 1Byte | 1Byte | NC or 15Byte | 2Byte |

  - Option：command control options

0x01：set up；

0x02：read；

Other values: invalid；

- Response format and status byte

set response：

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x005D | 1Byte | 1Byte | 1Byte | 2Byte |

get response：

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | Para[] |  |
| 0xCF | 0x00 | 0x0005D | 1Byte | 1Byte | 1Byte | NC or 15Byte | 2Byte |

Definition of each byte：

| field |  | byte | describe |
| --- | --- | --- | --- |
| STATUS |  | 1 | The command just returns the execution status. |
| Option |  | 1 | 0x01: set; 0x02: get |
| Para[] | RFIDPRO | 1 | 0x00: ISO 18000-6C; other values are invalid |
|  | STRATFREI | 2 | Integer part of MHz start frequency (see RFM_SET_FRE for details) |
|  | STRATFRED | 2 | Fractional part of MHz start frequency (see RFM_SET_FRE for details) |
|  | STEPFRE | 2 | Frequency step (KHz) (see RFM_SET_FRE for details) |
|  | CN | 1 | Number of channels (see RFM_SET_FRE for details) |
|  | Power | 1 | The output power of the reader. The range is 0 to 33. |
|  | Antenna | 1 | Antenna configuration information. The value of the corresponding Bit bit is 1, which means that the antenna is used, and the value is 0, which means that the antenna is not used; starting from the lower bit, the 0th bit indicates the No. 1 antenna, the 1st bit indicates the No. 2 antenna, and so on. |
|  | Region | 1 | National frequency range. |
|  | Addr | 1 | device address. |
|  | Baud | 1 | 0x00：9600； 0x01：19200；0x02：38400；0x03：57600；0x04：115200（默认）； |
|  | Buzzer | 1 | 0：Close；1：Open |
|  | Reserve | 1 | reserve |

2.2.16 RFM_MAXTEMPER_SET

- This command is used to set the maximum working temperature of the module. When the temperature reaches the set value, the system will be forced to sleep in multiples of 30S, and continue to work when the current temperature drops below the maximum working temperature by 5°C. The default value is 75°C.

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Max Value | Reserve |  |
| 0xCF | 0xFF | 0x0060 | 1Byte | 1Byte | NC or 1Byte | 2Byte |

  - MaxValue：Maximum temperature, unit: °C, the value range is 50~90 °C.

0x01：set up；

0x02：read；

Other values: invalid；

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0060 | 1Byte | 1Byte | 2Byte |

2.2.17 RFM_CURRTEMPER_GET

This command is used to read the current working temperature and maximum working temperature of the module.

- command format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x0061 | 1Byte | 2Byte |

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | CurValue | MaxValue |  |
| 0xCF | 0x00 | 0x0061 | 1Byte | 1Byte | 1Byte | 1Byte | 2Byte |

| field | byte | describe |
| --- | --- | --- |
| STATUS | 1 | 0x00: The command is executed successfully; 0x01: The parameter is wrong. |
| CurValue | 1 | Current working temperature, unit ℃ |
| MaxValue | 1 | Maximum working temperature, unit ℃ |

2.2.19 RFM_SET_GET_ANT_POWER

- This command is used to factory set the antenna port power compensation for calibration.

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Operate | AntPower |  |
| 0xCF | 0xFF | 0x0063 | 1Byte | 1Byte | NC or 8Byte | 2Byte |

  - Option：Command Control Options

0x01：set up；

0x02：read；

Other values: invalid；

- AntPower：Antenna power, eight bytes;

- Response format and status byte

set response：

| HEAD | ADDR | CMD | LEN | STATUS | Operate | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0063 | 1Byte | 1Byte | 1Byte | 2Byte |

get response：

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Operate | AntPower [] |  |  |
| 0xCF | 0x00 | 0x00063 | 1Byte | 1Byte | 1Byte | 9Byte | 2Byte |  |

| field |  | byte | describe |
| --- | --- | --- | --- |
| STATUS |  | 1 | 0x00: The command is executed successfully; 0x01: The parameter is wrong. |
| Operate |  | 1 | 0x01: set; 0x02: get |
| AntPower [] | Enable | 1 | Enable antenna power setting |
|  | AntPower [0] | 1 | - Antenna 1 power; unit: dBm, value range: [10, 33]dBm, others are invalid |
|  | AntPower [1] | 1 | Antenna 2 power |
|  | AntPower [2] | 1 | Antenna 3 power |
|  | AntPower [3] | 1 | Antenna 4 power |
|  | AntPower [4] | 1 | Antenna 5 power |
|  | AntPower [5] | 1 | Antenna 6 power |
|  | AntPower [6] | 1 | Antenna 7 power |
|  | AntPower [7] | 1 | Antenna 8 power |

2.2.21 RFM_SET_GET_DUTY_CYCLE

This command sets/reads the remote network port information, the device sends data like a remote ip, and the remote computer can monitor and receive data.

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | DutyPara |  |
| 0xCF | 0xFF | 0x0064 | 1Byte | 1Byte | NC or 2Byte | 2Byte |

  - Option：command control options

0x01：set up；

0x02：read；

Other values: invalid；

- Response format and status byte

set response：

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x005F | 1Byte | 1Byte | 1Byte | 2Byte |

get response：

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | DutyPara |  |
| 0xCF | 0x00 | 0x0005F | 1Byte | 1Byte | 1Byte | NC or 2Byte | 2Byte |

2.2.22 RFM_SET_ALL_PARAM

- This command is used to set all configurable parameters of the device. Users need to use this command to set all configurable parameters of the device before using the module to operate the label. If the user does not make this setting, the device will use the default setting when working.

This command can set the device to the active mode, and the returned data is in the RFM_INVENTORYISO_CONTINUE command format. It is recommended that the query interval be 50ms, otherwise the device will heat up quickly, and the card reading performance will be weakened if there is no good heat dissipation device.

- Command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Addr | RFIDPRO | Work Mode | Interface | Baudrate | WGSet | Ant | RfidFreq |
| 0xCF | 0xFF | 0x0071 | 0x19 | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 8Bytes |

| PAYLOAD |  |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RfidPower | InquiryArea | QValue | Session | AcsAddr | AcsDataLen | FilterTime | TriggerTime | BuzzerTime | Polling Interval |  |
| 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 2Bytes |

- Addr：The communication address of the device, the default is 0x00. This address cannot be 0xFF. If it is set to 0xFF, the read-write module will return parameter error information.

- RFIDPRO：Device radio frequency RFID protocol standard specification, 0x00: ISO 18000-6C; 0x01: GB/T 29768; 0x02: GJB 7377.1; currently only supports ISO 18000-6C.

- WorkMode：Working mode of the device, the default value is 0. Only supports answer mode and active mode

| WorkMode | Operating mode |
| --- | --- |
| 0 | answer mode |
| 1 | active mode |
| 2 | trigger mode |

- Interface：The communication interface of the device, the default value is 0x80, and the specific interpretation is as follows: Only RS232 is supported

| Interface | Interface Type |
| --- | --- |
| 0x80 | RS232 |
| 0x40 | RS485 |
| 0x20 | RJ45 |
| 0x10 | WiFi |

  - Baudrate：The serial port baud rate, the default value is 4, the specific interpretation is as follows:

| baudrate | actual baud rate |
| --- | --- |
| 0 | 9600bps |
| 1 | 19200 bps |
| 2 | 38400 bps |
| 3 | 57600 bps |
| 4 | 115200 bps |

  - WGSet：Configuration parameters of the Wiegand data output interface, the default value is 0x00, and the specific interpretation is as follows: Wiegand is not supported

| WGSet | Bit7 | Bit6 | Bit5 | Bit4 | Bit3 | Bit2 | Bit1 | Bit0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit definition | 0: Disable Wiegand output 1: Enable Wiegand output | 0：wg26 1：wg34 | 0：low position first 1：high position first | spare | spare | spare | spare | spare |

  - Ant：Antenna number of the device, which indicates the selected antenna by bit. The corresponding bit value is 1, which means that the antenna is used, and the value is 0, which means that the antenna is not used; 1 bit means antenna No. 2, and so on, up to 8 antennas can be represented; different modules support different antennas, depending on the specific situation; the default value is 0x01, which means antenna No. 1.

  - RfidFreq：The RFID frequency related parameters of the device are used to select the frequency band and the upper limit frequency point and lower limit frequency point in each frequency band. The length is 8Bytes. The specific parameters are as follows：

| RfidFreq |  |  |  |  |
| --- | --- | --- | --- | --- |
| REGION | STRATFREI | STRATFRED | STEPFRE | CN |
| 1 Byte | 2Byte | 2Byte | 2Byte | 1Byte |

Definition of each byte:

To set the frequency, first determine the frequency range according to REGION, and then refer to STRATFREI, STRATFRED, STEPFRE, and CN to calculate the specific frequency range.

| Field | Byte | Describe |
| --- | --- | --- |
| REGION | 1 | National frequency range: 0x00: User-defined according to requirements; 0x01：US [902.75~927.25] 0x02：Korea [917.1~923.5] 0x03：EU [865.1~868.1] 0x04：JAPAN [952.2~953.6] 0x05：MALAYSIA [919.5~922.5] 0x06：EU3 [865.7~867,5] 0x07：CHINA_BAND1 [840.125~844.875] 0x08：CHINA_BAND2 [920.125~924.875] |
| STRATFREI | 2 | The integer part of the megahertz start frequency；Such as 920.125MHz，STRATFREI = 920 = 0x0398，high byte=0x03，low byte=0x98； |
| STRATFRED | 2 | Fractional part of the megahertz start frequency；Such as 920.125MHz，STRATFRED =125，high byte=0x00，low byte=0x7D |
| STEPFRE | 2 | Frequency step (KHz), need to refer to the calculation formula of each frequency band；Such as 125KHz，STEPFRE =125，high byte=0x00，low byte=0x7D； |
| CN | 1 | number of channels； |

Frequency calculation formula: minimum frequency point：Fmin = STRATFREI + STRATFRED/1000  （unit：MHz）

maximum frequency point：Fmax = Fmin + STEPFRE*CN/1000 （单位：MHz）

Calculation formula for each frequency band：(The set frequency range must be within the national standard)

Chinese band2：		Fs = 920.125 + CN * 0.25 (MHz) in CN∈[0, 19]。

US band：				Fs = 902.75 + CN * 0.5 (MHz)  in CN∈[0,49]。

Korean band：			Fs = 917.1 + CN * 0.2 (MHz)  in CN∈[0, 31]。

EU band:				Fs = 865.1 + CN*0.2(MHz)  in CN∈[0, 14]。

Ukraine band:			Fs = 868.0 + CN*0.1(MHz)  in CN∈[0, 6]。

Peru band：			Fs = 916.2 + CN*0.9(MHz)  in CN∈[0, 11]。

Chinese band1：		Fs = 840.125 + CN * 0.25 (MHz)  in CN∈[0, 19]。

EU3 band：			Fs = 865.7 + CN * 0.6(MHz)  in CN∈[0, 3]。

US band3：			Fs = 902 + CN * 0.5 (MHz)  in CN∈[0,52]。

Taiwan band：			Fs = 922.25 + CN * 0.5 (MHz) in CN∈[0,11]。

  - RfidPower：The RFID output power of the device, the unit is: dBm, the value range is: [0, 30]dBm, other invalid。

  - InquiryArea：The memory area of the tag to be accessed by the device. 0x00: reserved area; 0x01 (default): EPC storage area; 0x02: TID storage area; 0x03: USER storage area; 0x04: EPC+TID; 0x05: EPC+USER; 0x06: EPC+TID+USER; other values are reserved. If other values appear in the command, a parameter error message will be returned。

  - QValue：询The initial Q value used when checking EPC tags, the setting of Q value should be that the number of tags in the field is approximately equal to 2Q. The default value of the Q value is 4, and the range of the Q value is 0 to 15. If other values appear in the command, a parameter error message will be returned.

  - Session: The Session value used when querying the EPC tag, the default value is 0, and the value range is [0, 3]. Other values will return a parameter error message.

| Session | note |
| --- | --- |
| 0 | Session uses S0 |
| 1 | Session uses S1 |
| 2 | Session uses S2 |
| 3 | Session uses S3 |

  - AcsAddr：The start address of the device to access the label storage area, unit: Byte, default value: 0x00: when accessing the EPC area, 0x00 indicates the start address of the EPC number segment except for the CRC and PC segments when accessing the EPC area; when accessing other storage areas, 0x00 Indicates the starting address of the storage area.

  - AcsDataLen：The data length of the device to access the label storage area, unit: Byte, default value: 0x00.

  - FilterTime: filter time, within this value time after successfully reading a tag data, filter out tags with the same data. The unit is: S, the value range is: [0, 255], others are invalid; the default value is 0, no filtering.

  - TriggerTime：The query time after the device receives the trigger signal, the unit is: S, the default value is 1, and the value range is: [0, 255], others are invalid.

  - BuzzerTime：The duration of the buzzer beeping after the device executes successfully, the unit is: 10ms, the value range is: [0, 255], others are invalid; the default is 1, when it is 0, it means the buzzer does not beep.

  - PollingInterval：Query interval time, unit: 10ms, value range: [0, 255], other invalid, default is 5.

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0xXX | 0x0071 | 0x01 | 1Byte | 2Byte |

  - STATUS：

  - 0x00：command executed successfully；

  - 0x01：Parameter error；

  - Other values: invalid；

2.2.23 RFM_GET_ALL_PARAM

This command is used to read the RF power currently set by the module. Users can use this command to view the power setting of the module before using the module to operate the label.

- Command format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x0072 | 0x00 | 2Byte |

- Response format and status byte

| HEAD | ADDR | CMD | LEN | PAYLOAD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | STATUS | Addr | RFIDPRO | Work Mode | Interface | Baudrate | WGSet | Ant | RfidFreq |
| 0xCF | 0xFF | 0x0071 | 0x1A | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 8Bytes |

| PAYLOAD |  |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RfidPower | InquiryArea | QValue | Session | AcsAddr | AcsDataLen | FilterTime | TriggerTime | BuzzerTime | Polling Interval |  |
| 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 2Bytes |

  - STATUS：

  - 0x00：command executed successfully；

  - Other values: invalid；

  - Refer to other parameters RFM_SET_ALL_PARAM。

| field |  | byte | describe |
| --- | --- | --- | --- |
| STATUS |  | 1 | 0x00: The command is executed successfully; 0x01: The parameter is wrong. |
| Option |  | 1 | 0x01: set; 0x02: get |
| DutyPara | num | 1 | Inventory wheel space interval times, for example, 0: no bye, 1: every time an inventory is taken; the default is 1; |
|  | time | 1 | Inventory free time, unit ms, default is 50; |

2.3 International standard (ISO 18000-6C) protocol-related commands

2.3.1 RFM_INVENTORYISO_CONTINUE

This command is to start an international standard multi-label inventory command, and the inventory function has an anti-collision algorithm.

This command can specify the inventory counting duration, if the input counting times is 0, it means that the tag will continue to be counted until the command to stop the inventory counting is received. During the inventory process, every time a tag is successfully counted, it will return the newly counted tag information through a command response with a "STATUS" value of "0".

When the inventory command is successfully executed, the command response with the "STATUS" value of "0x12" will be returned at last, which is used to notify that the inventory command has been executed.

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | InvType | InvParam |  |
| 0xCF | 0xFF | 0x0001 | 0x05 | 1Byte | 4Byte | 2Byte |

  - InvType：Inventory method：

0x00：According to the time inventory label, stop the inventory after the specified time continues or after receiving the order to stop the inventory;

0x01: Inventory according to the number of cycles, and stop the inventory after performing the specified number of polling or receiving the instruction to stop the inventory;

0x03：Custom wrapping (mask wrapping), (select and query parameters use RFM_SET_SELPRM and RFM_SET_QUERY_PARAM command setting), stop the inventory after the specified time continues or after receiving the command to stop the inventory;

  - InvParam ：Inventory method parameters:

1.If InvType is 0x01：

InvParam indicates the inventory time, the unit is: second, if the value is 0, it means that the inventory tag will continue until the command to stop the inventory is received;

2.If InvType is 0x02:

InvParam indicates the counting times, the unit is: times, the value must be greater than 0;

3.If InvType is 0x03:

InvParam indicates the inventory time, the unit is: second, the maximum time of this value is 255;

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | RSSI | Antenna | Channel | EPC LEN | EPC NUM |  |
| 0xCF | 0x00 | 0x0001 | 1Byte | 1Byte | 2Bytes | 1 Byte | 1 Byte | 1Byte | N Bytes | 2Byte |

Definition of each byte:

| field | byte | describe |
| --- | --- | --- |
| STATUS | 1 | 0x00: Successfully inventoried the tag, and the PAYLOAD contains the tag information; 0x01: Q, MemBank parameter values are wrong or Length and Mask data lengths are inconsistent; 0x02: The command execution failed due to an internal error in the module; 0x12: There is no inventory to the label or the entire inventory command is executed; 0x17: The label data exceeds the maximum transmission length of the serial port; Other values: void; |
| RSSI | 2 | The RSSI of the tag ACK response, the unit is 0.1dBm, a signed number, negative numbers use complement format; |
| Antenna | 1 | number of antennas |
| Channel | 1 | The tag data is received from which channel, the value starts from 0, 0 means channel 0, 1 means channel 1, and so on; |
| EPC LEN | 1 | The length of the tag's EPC number (bytes); |
| EPC NUM | N Byte | EPC number of the label; |

2.3.2 RFM_INVENTORY_STOP

This command users to actively stop the international standard multi-label anti-collision inventory process.

- command format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x0002 | 0x00 | 2Byte |

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0002 | 0x01 | 1Byte | 2Byte |

 STATUS：This command will only return successful execution (value 0x00), other values are invalid;

2.3.3 RFM_READISO_TAG

This command frame is for the module to read the data in the label storage area of the international standard protocol.

- Before calling this method, please set the inventory condition (for selecting tags) through the RFM_SETISO_SELECTMASK command. When performing international standard protocol tag data operations (reading tag data, writing tag data, locking tags, deactivating tags), the tag will return the tag operation status, and the module’s response contains a byte of tag operation status. See Appendix A for the definition of tag operation status

- command format

| HEAD | ADDR | CMD | LEN |  | PAYLOAD |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | AccPwd | MemBank | WordPtr | WordCount |  |
| 0xCF | 0xFF | 0x0003 | 1Byte | 1Byte | 4 Bytes | 1Byte | 2 Bytes | 1 Bytes | 2Byte |

Definition of each field：

| field | byte | describe |
| --- | --- | --- |
| Option | 1 | Command option (this field is not used, invalid, default 0x00); |
| AccPwd | 4 | Access password, used for the label to enter the security state, the default is 0x00000000; |
| MemBank | 1 | The storage area of the tag to be read, the list of values is as follows: 0x00: Reserved; 0x01: EPC; 0x02: TID; 0x03: User; |
| WordPtr | 2 | Point to the read start address (word) of the logical storage area; |
| WordCount | 1 | The number of words to be read; - 1. When WordCount is equal to 0 and MemBank is not 01 access area, the data content returned by the tag is from the start address defined by WordPtr to the end of the storage area. - When WordCount is equal to 0 and MemBank is equal to 01 access area, the data content returned by the tag is from the start address defined by WordPtr to the end of the EPC length specified by the first 5 bits of PC. |

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TagStatus | Antenna | CRC | PC | EPC LEN | EPC NUM | WordCount | Data |  |
| 0xCF | 0x00 | 0x0003 | 1Byte | 1Byte | 1Byte | 1Byte | 2Byte | 2Bytes | 1Byte | N Bytes | 1Bytes | N-Byte | 2Byte |

Each field definition:

Judge whether the command is successfully executed according to STATUS and TagStatus.

| field | byte | describe |
| --- | --- | --- |
| STATUS | 1 | Refer to Table A-6 STATUS definition |
| TagStatus | 1 | The operation status returned by the tag, see Appendix A for specific definitions; |
| Antenna | 1 | which antenna to read from |
| CRC | 2 | CRC data in tag response data |
| PC | 2 | PC data in tag response data |
| EPC LEN | 1 | The length of the tag's EPC number (bytes); |
| EPC NUM | N Byte | EPC number of the tag; |
| WordCount | 1 | The number of tag data words successfully read; |
| Data | N Byte | The tag data successfully read, the length is WordCount×2 bytes; |

2.3.4 RFM_WRITEISO_TAG

This command writes data to the label storage area for the module.

Before calling this method, please set the inventory condition (for selecting tags) through the RFM_SETISO_SELECTMASK command1

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | AccPwd | MemBank | WordPtr | WordCount | Data |  |
| 0xCF | 0xFF | 0x0004 | 1Byte | 1Byte | 4Byte | 1Byte | 2Byte | 1Byte | N-Byte | 2Byte |

Definition of each field：

| field | byte | describe |
| --- | --- | --- |
| Option | 1 | Command option (this field is not used, invalid, default 0x00) ； |
| AccPwd | 4 | Access password, used for the label to enter the security state, the default is 0x00000000； |
| MemBank | 1 | The storage area of the tag to be read, the list of values is as follows: 0x00：Reserved； 0x01：EPC； 0x02：TID； 0x03：User； |
| WordPtr | 2 | Points to the read start address (word) of the logical memory area； |
| WordCount | 1 | The number of data words that need to be written to the tag (one word is two bytes), must be greater than 0; |
| Data | N-Byte | The data to be written to the tag must be an integer multiple of words, and the length is 1 to WordCount words |

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS |  |  | PAYLOAD |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TagStatus | Antenna | CRC | PC | EPC LEN | EPC NUM |  |
| 0xCF | 0x00 | 0x0004 | 1Byte | 1Byte | 1Byte | 1Byte | 2Byte | 2Bytes | 1Byte | N Bytes | 2Byte |

  - TagStatus：The operation status returned by the tag is defined in Appendix A. Only when the value of the STATUS field is 0, this field will be included in the response data;

Definition of each field：

Judging whether the command is executed successfully according to STATUS and TagStatus.

| field | byte | describe |
| --- | --- | --- |
| STATUS | 1 | Refer to Table A-6 STATUS definition |
| TagStatus | 1 | The operation status returned by the tag, see Appendix A for specific definitions; |
| Antenna | 1 | From which antenna port the tag data is received, the value range is: 1~4, indicating antenna 1~4 respectively |
| CRC | 2 | CRC data in tag response data |
| PC | 2 | PC data in tag response data |
| EPC LEN | 1 | The length of the tag's EPC number (bytes); |
| EPC NUM | N Byte | EPC number of the label; |

2.3.5 RFM_LOCKISO_TAG

This command locks the specified storage data area of the specified international standard protocol label.

Before calling this method, please set the inventory condition (for selecting tags) through the RFM_SETISO_SELECTMASK command.

- command format

| HEAD | ADDR | CMD | LEN |  | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | AccPwd | Area | Action |  |
| 0xCF | 0xFF | 0x0005 | 0x06 | 4Byte | 1Byte | 1Byte | 1Byte |

Definition of each field：

| field | byte | describe |
| --- | --- | --- |
| AccPwd | 4 | Access password, used for the label to enter the security state, the default is 0x00000000 |
| Area | 1 | The area that needs to be locked, the list of values is as follows: 0x00: Inactivate password area; 0x01: Access password area; 0x02: EPC; 0x03: TID; 0x04: User; |
| Action | 1 | Lock operation type, the list of values is as follows: 0x00: open; 0x01: permanently open; 0x02: locked; 0x03: permanently locked; |

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS |  | PAYLOAD |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TagStatus | Antenna | CRC | PC | EPC LEN | EPC NUM |  |
| 0xCF | 0x00 | 0x0005 | 1Byte | 1Byte | 1Byte | 1Byte | 2Bytes | 2Bytes | 1Byte | N Bytes | 1Byte |

Definition of each field：

Judge whether the command is successfully executed according to STATUS and TagStatus.

| field | byte | describe |
| --- | --- | --- |
| STATUS | 1 | Refer to Table A-6 STATUS definition |
| TagStatus | 1 | The operation status returned by the tag, see Appendix A for specific definitions; |
| Antenna | 1 | From which antenna port the tag data is received, the value range is: 1~4, indicating antenna 1~4 respectively |
| CRC | 2 | CRC data in tag response data |
| PC | 2 | PC data in tag response data |
| EPC LEN | 1 | The length of the tag's EPC number (bytes); |
| EPC NUM | N Byte | EPC number of the label; |

2.3.6 RFM_KLENLISO_TAG

This command kills tags for modules.

Before calling this method, please set the inventory condition (for selecting tags) through the RFM_SETISO_SELECTMASK command.

Inactivation, only non-zero inactivation passwords are supported.

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD | CHECK |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | KLENlPwd |  |
| 0xCF | 0xFF | 0x0006 | 0x04 | 4Byte | 1Byte |

  - KLENlPwd：KLENl command password；

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS |  |  | PAYLOAD |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TagStatus | Antenna | CRC | PC | EPC LEN | EPC NUM |  |
| 0xCF | 0x00 | 0x0006 | 1Byte | 1Byte | 1Byte | 1Byte | 2Byte | 2Byte | 1Byte | N Bytes | 1Byte |

Definition of each field：

Judge whether the command is successfully executed according to STATUS and TagStatus.

| field | byte | describe |
| --- | --- | --- |
| STATUS | 1 | Refer to Table A-6 STATUS definition |
| TagStatus | 1 | The operation status returned by the tag, see Appendix A for specific definitions; |
| Antenna | 1 | From which antenna port the tag data is received, the value range is: 1~4, indicating antenna 1~4 respectively |
| CRC | 2 | CRC data in tag response data |
| PC | 2 | PC data in tag response data |
| EPC LEN | 1 | The length of the tag's EPC number (bytes); |
| EPC NUM | N Byte | EPC number of the label; |

2.3.7 RFM_SETISO_SELECTMASK

This command is used to set the tag matching information required for tag operations (read, write, lock and kill).

- command format

| HEAD | ADDR | CMD | LEN |  | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Pointer | Length | Mask |  |
| 0xCF | 0xFF | 0x0007 | 1Byte | 2Bytes | 1Byte | N-Bytes | 2Byte |

  - Pointer：Reserved, the default value is 0x0000；

  - Length：The bit length of the EPC number that needs to be matched, the default value is 0x00; if the Length is 0, it means no tag is specified, multi-tag operation.

  - Mask：The data that needs to be matched, the effective data length is Length bits, if the Length is an odd number, you need to add 0 to the lower bits of the mask;

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0007 | 0x01 | 1Byte | 2Byte |

  - STATUS：

  - 0x00：command executed successfully；

  - 0x01：Parameter error；

  - Other values: invalid；

2.3.8 RFM_SET_ISO_Q

This command is used to set the Q value of the international standard protocol (ISO 18000-6C) currently set by the module.

- Command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | CALEN_PRM | Reserved |  |
| 0xCF | 0xFF | 0x0008 | 0x02 | 1byte | 1byte | 2Byte |

  - CALEN_PRM: Q value, value range [0, 15], no need to set the default to 4; the larger the value, the better the effect when multi-label;

  - Reserved：unused, reserved;

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0008 | 1Byte | 1Byte | 2Byte |

  - STATUS：This command will only return successful execution (value 0x00), other values are invalid;

2.3.9 RFM_GET_ISO_Q

This command is used to obtain the Q value of the international standard protocol (ISO 18000-6C) currently set by the module

- Command format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x0009 | 0x00 | 2Byte |

- Response format and status byte

|  | HEAD | ADDR | CMD | LEN | STATUS | CALEN_PRM | CHECK |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0xCF | 0x00 | 0x0009 | 1Byte | 1Byte | 1Byte | 2Byte |  |

STATUS：This command will only return successful execution (value 0x00), other values are invalid；

CALEN_PRM：The obtained Q value, the value range is [0, 15]；

2.3.10 RFM_SET_SELPRM

- This command is used to set the Select command parameters of the specified protocol of the module, including "MemBank", "Target", "Action", "Pointer", "Length", and "Mask". For the specific application and meaning of each parameter, please refer to the protocol description

- Command format

| HEAD | ADDR | CMD | LEN |  |  |  | PAYLOAD |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Protocal | Target | Truncated | Action | Membank | Pointer | Length | Mask |  |
| 0xCF | 0xFF | 0x0010 | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 2Bytes | 1Byte | N-Bytes | 2Byte |

  - Protocal：agreement type

0x00：ISO

  - Target：Select modify target，

0x00：S0 ;0x01：S1 ;0x02：S2 ;0x03：S3 ;0x04：SL ;0x05-0xFF：reserve。

  - Truncated：Whether to truncate，

0x00: no truncation; 0x01: truncation。

  - Action：Select matching criteria rules

| Action | ISO |
| --- | --- |
| 0x00 | Set as SL or A if eligible, set as not SL or B if not eligible |
| 0x01 | Eligibility is set to SL or A |
| 0x02 | Not eligible set to non-SL or B |
| 0x03 | Meet the conditions to reverse SL or A->B, B->A |
| 0x04 | Set as not SL or B if eligible, and as SL or A if not eligible |
| 0x05 | Eligibility set to non-SL or B |
| 0x06 | Not eligible set to SL or A |
| 0x07 | Invert SL or A->B, B->A if the conditions are not met |

  - MemBank：The store where the selected and matched data resides

| Membank | ISO | paraphrase | Remark |
| --- | --- | --- | --- |
| 0x00 | RFU | Label information area | Label information area |
| 0x10 | EPC | Coding region | Coding region |
| 0x20 | TID | Safe place | Safe place |
| 0x30 | User | user area | user subarea 0 |

  - Pointer: The starting address of the match, in bits

  - Length: the matching mask length, in bits

  - Mask: The data that needs to be matched, the effective data length is Length bits, if Length is an odd number, you need to add 0 to the lower bits of the mask;

- Response format and status byte

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Status | Protocal |  |
| 0xCF | 0x00 | 0x0010 | 1Byte | 1Byte | 1Byte | 2Byte |

  - Status：command response code

0x00：command executed successfully；

0x01：Parameter error；

Other values: invalid；

  - Protocal：agreement type

0x00：ISO

2.3.11 RFM_GET_SELPRM

This command is used to obtain the Select/Sort command parameters of the specified protocol of the module, including "MemBank", "Target", "Action", "Pointer", "Length", and "Mask" The specific application and meaning of each parameter refer to the protocol illustrate

- Command format

| HEAD | ADDR | CMD | LEN | PAYLOAD | CHECK |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Protocal |  |
| 0xCF | 0xFF | 0x0011 | 1Byte | 1Byte | 2Byte |

  - Protocal：Protocal type

0x00：ISO

- Response format and status byte

| HEAD | ADDR | CMD | LEN |  |  |  |  | PAYLOAD |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Status | Protocal | Truncated | Target | Action | Membank | Pointer | Length | Mask |  |
| 0xCF | 0x00 | 0x0011 | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 2Bytes | 1Byte | N-Bytes | 2Byte |

  - Status：Command response code

0x00：The command is executed successfully; 0x01: parameter error; other values: invalid;

  - Protocal：Protocol type

0x00：ISO

  - Target：Select modify target，

0x00：S0 0x01：S1 0x02：S2 0x03：S3 0x04：SL 0x05-0xFF：reserve

  - Truncated：Whether to truncate，

0x00：Do not truncate；0x01：cut off。

  - Action：Select matching criteria rules

| Action | ISO |
| --- | --- |
| 0x00 | Set as SL or A if eligible, set as not SL or B if not eligible |
| 0x01 | Eligibility is set to SL or A |
| 0x02 | Not eligible set to non-SL or B |
| 0x03 | Meet the conditions to reverse SL or A->B, B->A |
| 0x04 | Set as not SL or B if eligible, and as SL or A if not eligible |
| 0x05 | Eligibility set to non-SL or B |
| 0x06 | Not eligible set to SL or A |
| 0x07 | Invert SL or A->B, B->A if the conditions are not met |

  - MemBank：Select and match the storage area where the data is located

| Membank | ISO | paraphrase | Remark |
| --- | --- | --- | --- |
| 0x00 | RFU | Label information area | Label information area |
| 0x10 | EPC | Coding region | Coding region |
| 0x20 | TID | Safe place | Safe place |
| 0x30 | User | user area | user subarea 0 |

  - Pointer：The starting address of the match, in bits

  - Length：Matching mask length, in bits

  - Mask：The data that needs to be matched, the effective data length is Length bits, if Length is an odd number, you need to add 0 to the lower bits of the mask；

2.3.12 RFM_SET_QUERY_PARAM

- This command is used to set the Query command parameters of the specified protocol of the module, including "Sel", "Session", and "Target". For the specific application and meaning of each parameter, please refer to the protocol description

- Command format

| HEAD | ADDR | CMD | LEN |  | PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Protocal | Sel | Session | Target |  |
| 0xCF | 0xFF | 0x0012 | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 2Byte |

  - Protocal：Protocol type

0x00：ISO

  - Sel：Specifies the tag for the inventory response

0x00 : ALL;  0x01:ALL;  0x02: ~SL;  0x03: SL。

  - Session：Specifies the section for the inventory tag

0x00：S0 ；0x01：S1 ；0x02：S2 ；0x03：S3

  - Target：Inventory logo

0x00：A

0x01：B

- Response format and status byte

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Status | Protocal |  |
| 0xCF | 0x00 | 0x0012 | 0x02 | 1Byte | 1Byte | 2Byte |

  - Status：command response code

0x00：The command is executed successfully; 0x01: parameter error; other values: invalid;

  - Protocal：Protocol type

0x00：ISO

2.3.13 RFM_GET_QUERY_PARAM

This command is used to get the Query instruction parameters of the specified protocol of the module, including "Sel", "Session", and "Target". For the specific application and meaning of each parameter, please refer to the protocol description。

- Command format

| HEAD | ADDR | CMD | LEN | PAYLOAD | CHECK |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Protocal |  |
| 0xCF | 0xFF | 0x0013 | 1Byte | 1Byte | 2Byte |

  - Protocal：Protocol type

0x00：ISO

- Response format and status byte

| HEAD | ADDR | CMD | LEN |  |  | PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Status | Protocal | Sel | Session | Target |  |
| 0xCF | 0x00 | 0x0013 | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 2Byte |

  - Status：command response code

  - 0x00: command executed successfully;  0x01: parameter error;

  - other values: invalid;

  - Protocal：Protocol type

0x00：ISO

  - Sel：Specifies the tag for the inventory response

0x00 : ALL;  0x01:ALL;  0x02: ~SL;  0x03: SL。

  - Session：Specify the section of the inventory label, this field is valid for ISO and national standard, and the national standard does not use

0x00：S0 ；0x01：S1 ；0x02：S2 ；0x03：S3

  - Target：Inventory logo

0x00：A

0x01：B

2.3.15 RFM_INVENTORYISO_RANGE_DATA

This command is used to set the output data range during inventory, which is convenient for analysis. This command needs to be issued before each inventory。

It has the function of controlling the output length. If the length is set to 0, the full length will be defaulted. If the set length exceeds the length of the EPC number, the actual length will be output.

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Operate | RangePara |  |
| 0xCF | 0xFF | 0x0018 | 1Byte | 1Byte | NC or 4Byte | 2Byte |

  - Option：command control options

0x01：set up；

0x02：read；

Other values: invalid；

- DeltaPara：Power compensation value, four bytes；

Response format and status byte

Set response：

| HEAD | ADDR | CMD | LEN | STATUS | Operate | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0018 | 1Byte | 1Byte | 1Byte | 2Byte |

Get response：

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Operate | RangePara [] |  |
| 0xCF | 0x00 | 0x0018 | 1Byte | 1Byte | 1Byte | 4Byte | 2Byte |

| Field |  | Byte | Describe |
| --- | --- | --- | --- |
| STATUS |  | 1 | 0x00: The command is executed successfully; 0x01: The parameter is wrong. |
| Operate |  | 1 | 0x01: set; 0x02: get |
| RangePara [] | StartAddr | 1 | Output the starting address of the EPC number (byte); default: 0x00 |
|  | Len | 1 | Length of output data (byte), default 00: all output; |
|  | Reserve | 1 | reserve |
|  | Reserve | 1 | reserve |

Appendix A. Operational Status Returned by Tags

The operating status code returned by the tag has a total of 8 bits, see Table A-1。

Table A-1 Operation status returned by tags

| operating state code | operating state | describe | error priority |
| --- | --- | --- | --- |
| 0x14 | Tag response timed out |  |  |
| 0x81 | other errors | Unknown error returned by tag |  |
| 0x82 | storage area overflow | target store does not exist |  |
| 0x83 | storage lock | Write operations to memory areas that are locked as non-writable |  |
| 0x84 | Insufficient power | Operation or erase operation, read operation on the memory area that is locked as unreadable |  |
| 0x85 | non-specific error | The tag does not have enough energy to complete the operation |  |
|  | operating state |  |  |

Appendix B. CRC16 check reference C code

As follows：

#define PRESET_VALUE 0xFFFF

#define POLYNOMIAL  0x8408

unsigned int uiCrc16Cal(unsigned char const  * pucY, unsigned char ucX)

{

unsigned char ucI,ucJ;

unsigned short int  uiCrcValue = PRESET_VALUE;

for(ucI = 0; ucI < ucX; ucI++)

{

uiCrcValue = uiCrcValue ^ *(pucY + ucI);

for(ucJ = 0; ucJ < 8; ucJ++)

{

if(uiCrcValue & 0x0001)

{

uiCrcValue = (uiCrcValue >> 1) ^ POLYNOMIAL;

}

else

{

uiCrcValue = (uiCrcValue >> 1);

}

}

}

return uiCrcValue;

}

Appendix C. Definition of STATUS。

Table A-6 STATUS definition

| STATUS | Wrong description |
| --- | --- |
| 0x00 | Successful execution (here only means that the module has successfully received the tag response data, if there is a tag execution status in the tag response, it should be further judged whether the tag execution status is correct） |
| 0x01 | The parameter value is wrong or out of bounds, or the module does not support the parameter value |
| 0x02 | Command execution failed due to an internal error in the module (set frequency or set power) |
| 0x03 | reserve |
| 0x12 | The entire inventory command is executed |
| 0x13 | Not counted to the label |
| 0x14 | Tag response timed out |
| 0x15 | Demod tag response error |
| 0x16 | Protocol authentication failed |
| B |  |
| 0xFF | no more data |